HMI Color Philosophy for Steuerung3D Supervisor

Draft • 19 March 2026

Core rule: red means live danger, yellow means the protective reaction is active, blue means operator action is required, green means healthy/ready, and grey means neutral or informational.

# Purpose

This note defines a practical and standards-informed color philosophy for the Steuerung3D Supervisor and related HMIs. The aim is to keep routine screens visually calm, make abnormal situations immediately legible, and stay consistent across Supervisor, DenSi/HiP-related views, and later kinematic or visualization overlays.

# Semantic decision

The key project decision is to separate hazard from protective reaction. An emergency stop is not the hazard itself; it is the machine’s protective response to a hazard. That means ESTOP_ACTIVE should normally read as a protective or inhibited state, not as the same category as live uncontrolled danger.

# Semantic palette

| Bucket | Use for | Typical states | Accent | Soft background |
| --- | --- | --- | --- | --- |
| Danger | Hazard is present now | FAULT_MOVING, UNSAFE_MOTION, RUNAWAY, DRIVE_FAULT_WITH_ENABLE | #C63B3B | #F8D7DA |
| Protective / inhibited | Safety reaction active | ESTOP_ACTIVE, SAFETY_STOP, STO_ACTIVE, INTERLOCK_OPEN, INHIBITED | #D6A600 | #FFF3CD |
| Action required | Human must do something | RESET_REQUIRED, ACK_REQUIRED, RESYNC_REQUIRED, HOME_REQUIRED, CLAIM_REQUIRED | #2F6DB2 | #DCEBFA |
| Healthy / ready | Normal healthy condition | READY, SYNCED, OP, HEALTHY, REMOTE_READY | #4E8B57 | #DDEEDF |
| Neutral / informational | Informational only | BOOT, INIT, OFFLINE, IDLE, SIM, UNKNOWN | #7A7F87 | #EEF1F4 |

# Canonical rules

Rule A — E-stop is yellow, not red. ESTOP_ACTIVE belongs to the protective/inhibited bucket. It indicates that the machine has gone into a safety reaction.

Rule B — danger without protective reaction is red. Fault while motion remains enabled, uncontrolled motion, or other unsafe motion conditions belong to the danger bucket.

Rule C — recovery and handshakes are blue. Reset, acknowledge, re-home, re-sync, or claim steps belong to the action-required bucket.

Rule D — routine operation stays visually quiet. Strong saturated color is reserved for abnormal situations and operator-attention states.

# Application in the current Supervisor

Keep rows neutral by default.

Use the axis column for the persistent axis identity accent.

Use the phase/state cell for the semantic state color.

Allow subtle whole-row tinting only for red danger, yellow protective stop, and blue action-required states.

Always pair color with explicit text, iconography, tooltip, border treatment, or disabled/enabled affordance.

# Suggested state mapping

| Bucket | Mapped states |
| --- | --- |
| Danger | FAULT_MOVING, UNSAFE_MOTION, RUNAWAY, OVERSPEED_UNCONTROLLED, MOTION_WITH_SAFETY_VIOLATION |
| Protective / inhibited | ESTOP, ESTOP_ACTIVE, SAFETY_STOP, SAFETY_CHAIN_OPEN, STO_ACTIVE, INHIBITED |
| Action required | RESET_REQUIRED, ACK_REQUIRED, RESYNC_REQUIRED, HOME_REQUIRED, CLAIM_REQUIRED, ENABLE_REQUIRED |
| Healthy / ready | READY, SYNCED, OP, HEALTHY, REMOTE_READY, LOCAL_READY |
| Neutral / informational | BOOT, INIT, OFFLINE, DISCONNECTED, IDLE, SIM, UNKNOWN |

# References used

ISO 13850:2015 — Safety of machinery — Emergency stop function — Principles for design.

W3C WCAG 2.2 — Success Criterion 1.4.1 Use of Color.

EN/IEC 60204-1 and IEC 60073 color semantics summary as cited in Siemens functional safety terminology overview.

Note: This document defines the project-specific HMI interpretation for Steuerung3D. Physical operator devices still follow their own hardware standards and device-color conventions.